SOP5 – Channel Flood Risk Categorisation & Works Plan

Year__________ IDB_________________________ Operation_____________________________

| Channel Point Ref | Category: | Reasoning: | Approach: | YR1 | YR2 | YR3 | Notes & records: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Example (Flail mowing) |  |  |  |  |  |  |  |
| 1-7 | High | Flashy from housing estate run-off at all times of year. Has been bank-full in winter events and some high rainfall events in summer. Need to get water through quickly so it doesn’t back up and flood parking area. | FMe | Both | Both | Both | Cut before 15th Sept for harvest fete. |
| 8-14 | Medium | Never overtopped but key to conveyance in winter through district. Wouldn’t want to change anything that may slow things as upstream can get bank-full and floods Finch’s yard. | FMe | RB | LB | RB | WV definitely present. Follow species compliance standards carefully. |
| 25-37 | Low | Pump drain so wide. Plenty of capacity and links to adjacent nature reserve. Keep costs down and cut alternate sides | FMc | LB | RB | LB | Just cut edges of bramble scrub at end of Jacksons field to keep it same size. |

| Channel Point Ref | Category: | Reasoning: | Approach: | YR1 | YR2 | YR3 | Notes & records: |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Channel Point Ref | Category: | Reasoning: | Approach: | YR1 | YR2 | YR3 | Notes & records: |
| --- | --- | --- | --- | --- | --- | --- | --- |

Appendix I

| Operational Planning Information IDB Channel Risk Categories by Drainage Value/Seasonal Flood Risk |  |  |
| --- | --- | --- |
| Channel Categorisation | Justification: |  |  |
| High Risk (Category A) | Key to managing water levels across a wide local area in all seasons. Strong established need to maximise capacity and conveyance at all times. High level carriers and IDB main drains in more developed catchments with high residential, commercial and/or infrastructure risk. Complex interconnectivity with other channels. Normally at least annual bank and marginal vegetation cuts to allow for safety inspections & regular in-channel weed control. |  |  |
| Moderate Risk (Category B) | Important for managing seasonally high flows in less developed areas. Maintenance of in-stream, bank and marginal vegetation on a regular/rolling needs-basis to maintain required capacity and conveyance, and to allow for regular (but not necessarily annual) inspections to ensure good condition. |  |  |
| Low Risk (Category C) | Important for alleviation of seasonal waterlogging or agricultural drought in rural, less-developed areas, and to contribute to overall system capacity when needed. Vegetation managed occasionally if needed i.e. where it significantly impedes flow and capacity and for biodiversity value. |  |  |
| Note: These are suggested categorisations and justifications. It may be more appropriate for an IDB to add more categories or to replace these with other more relevant definitions for their own channels. |  |  |  |